[image: image1.jpg]dyson airblade




GUIDE SPECIFICATIONS


SECTION 10 28 13.14
ELECTRIC HAND DRYERS
PART 1 -  GENERAL

1.1 CONDITIONS AND REQUIREMENTS

A. The General Conditions, Supplementary Conditions, and Division 01 – General Requirements apply.

1.2 SECTION INCLUDES

A. Electric hand dryers.
1.3 RELATED SECTIONS

Specifier Note:  In this article, specify work specified in other sections that is related to work of this section.
A. Division 26 - Electrical:  Electrical systems and components.

Specifier Note:  The following paragraph is a sample that may be used in this article.  Add to or delete from the following as appropriate for the specific project.
B. Section [xxxxx] – [Section Title]:  [Include brief description of work specified in another section that is related to the work of this section.]
1.4 SUBMITTALS

Specifier Note:  In this article, specify various types of data to be furnished by the contractor before, during, or after construction. Topics included in this article are:  product data, shop drawings, samples, design data, test reports, certificates, manufacturers’ instructions, manufacturers’ field reports, qualification statements, and closeout submittals.
A. Submit under provisions of Section [01 33 00] [______].

B. Product Data:  Provide construction details, dimensions, anchoring and mounting requirements, material and finish descriptions, electrical requirements, and manufacturer's warranty.
C. Operation and Maintenance Data:  Provide for electric hand dryers to include in maintenance manuals.
D. Warranty:  Provide sample of manufacturer’s standard warranty for parts and labor.
1.5 QUALITY ASSURANCE

Specifier Note:  In this article, describe qualifications, regulatory requirements, certifications, field samples, mock-ups, and pre-installation meetings.
A. Product Certification:

1. ETL listed in accordance with UL 507.

2. Compliant with National Sanitation Foundation International (NSF) Protocol P335 "Hygienic Commercial Hand Dryers".

3. Certified non-food "food safe" product by HACCP International Endorsement Program.
B. Electrical Components, Devices, and Accessories:  Listed and labeled in accordance with NFPA 70, by a qualified testing agency, and marked for intended location and application.

1.6 DELIVERY, STORAGE AND HANDLING

A. Deliver, store, and handle electric hand dryers in manufacturer's protective packaging.
B. Store electric hand dryers off of ground, under cover, and in a dry location.  Handle according to manufacturer's written recommendations to prevent damage, deterioration, or soiling.
1.7 COORDINATION
A. Coordinate locations of electric hand dryers with other work to prevent interference with clearances required for access, and for proper installation, adjustment, operation, cleaning, and servicing of electric hand dryers.
1.8 WARRANTY

A. Manufacturer's Standard Warranty:  Manufacturer's standard form in which manufacturer agrees to repair, restore, or replace defective electric hand dryer components and labor within specified warranty period.

1. Warranty Period:  One (1) year limited for labor and five (5) years for parts.

PART 2 -  PRODUCTS

2.1 MANUFACTURERS
A. Basis-of-Design Product:  Subject to compliance with requirements, provide the electric Dyson Airblade dB hand dryers manufactured by Dyson Inc., 600 W. Chicago Avenue, Suite 275, Chicago, IL 60654; 888-397-6622, http://www.dyson.com/hand-dryers  or comparable product by one of the following:
1. Mitsubishi Electric; Jet Towel.

2. ASI Roval; TRI-Umph 20200.
B. Substitutions will be considered under provisions of Section 01 25 00.
2.2 ELECTRIC HAND DRYERS
Specifier Note:  The Dyson Airblade dB model replaces the previous AB04 model.  In the event that an earlier version of this specification was used on a project please note the update in model numbers on those projects.
Specifier Note:  For Section A in the subparagraph below, retain the first option if the low voltage gray finish is specified; retain the second option if the low voltage white finish is specified; retain the third option if the high voltage gray finish is specified; or retain the fourth option if the high voltage white finish is retained. ; If both colors or voltages are being used on a project, retain all individual options and indicate locations on the Drawings. For Point 9 in the subparagraph below, retain the first option if low voltage is specified; retain the second option if high voltage is specified.
A. Electric Hand Dryers:  The electric Dyson Airblade dB hand dryer (Model AB14); Item No. [301853-01 (gray LV)], [301854-01 (white LV)], [304663-01 (gray HV)] or [304664-01 (white HV)].
Mounting:  Surface mounted on pressed galvanized steel backplate.
Construction:  Polycarbonate-ABS casing with anti-microbial molded additive in facia and blades.  Anti-tamper M6 exterior machine screws.  Water ingress protection to IP35.
Bacterial Reduction Rate from External Surfaces:  Up to 99.9 percent over 24 hours.

Color Finish:  [Gray] [White].
Light Reflective Value:  [Gray - 28.3] [White - 77.4].

Filtration:  99.97 percent at 0.3 microns particulate efficiency HEPA filter with anti-microbial coating.
Operation:  Touch-free infra-red activation.

a. Hand dry time:  12 seconds

b. Airspeed at nozzle:  420 mph

c. Operation Lock-out Period:  30 seconds

d. Operating Airflow:  Up to 9.25 gallons/second
e. Rated Operating Noise Power:  81 db(A)

2. Motor:  Dyson Digital Motor (DDM), V4 switched reluctance brushless DC type; 92,000 rpm motor speed; less than 0.5 watt standby power consumption.
3. Electrical Requirements:  [110-127 V AC, 50 & 60 Hz, 1400 W] [200-240 V AC, 50 & 60 Hz, 1400 W].
PART 3 -  EXECUTION

3.1 EXAMINATION

A. Verify availability and characteristics of electrical power.  Drill minimum two (2) inch diameter holes for electrical service entrance through backplate.
B. Do not begin installation until substrates are complete and ready for installation of electric hand dryers.
3.2 INSTALLATION

A. Locate and install mounting bracket in accordance with manufacturer’s written instructions.  Use minimum 0.25-inch anchors to mount bracket.  Mount electric hand dryer at height above finished floor recommended by manufacturer.
B. Install electric hand dryer in accordance with manufacturer’s written instructions, using fasteners appropriate to substrate indicated and recommended by manufacturer.  Install electric hand dryers level, plumb, and firmly anchored in locations and at heights indicated.
3.3 CLEANING AND PROTECTION

A. Adjust electric hand dryers for smooth operation.  Replace damaged or defective components.

B. Remove protective coverings from finished surfaces.

C. Clean exposed surfaces using materials and methods recommended by manufacturer.

END OF SECTION

© 2017 Dyson Airblade™.  All Rights Reserved.

A copyright license to reproduce this specification is hereby granted 

to non-manufacturing engineers, architects and specification writers.
Rev 10/13
Project Name
Project No.
Date

Project Location
10 28 13.14-1
Electric Hand Dryers

[image: image1.jpg]